## WHAT THIS MEANS FOR YOU

## (Subject to Full Compliance with Permitted Development Legislation)

Based on available planning data, Permitted Development (PD) rights appear to apply to this property.

This means certain types of development may be possible without a full planning application, provided the proposal complies strictly with national legislation and any site-specific restrictions.

The following represents the most common forms of development that may be achievable under PD rules. Each is subject to detailed design review and statutory limits.

## 1. LOFT CONVERSIONS (most common PD use)

A loft conversion may be possible under PD, subject to:

- Maximum additional roof volume of:

  - 40 cubic metres (terraced or semi-detached house)

  - 50 cubic metres (detached house)

- No extension beyond the existing roof ridge height

- No extension beyond the front roof slope facing the highway

- Materials similar in appearance to the existing dwelling

Note: Design details, roof form, previous extensions and local restrictions can affect eligibility.

## 2. Single-Storey Rear Extensions

A single-storey rear extension may be possible, subject to:

- Maximum depth of:

  - 3 metres beyond the original rear wall (terrace or semi-detached house)

  - 4 metres beyond the original rear wall (detached house)

Under the Larger Home Extension Scheme (subject to neighbour consultation and prior approval process):

- Up to 6 metres (terrace/semi)

- Up to 8 metres (detached)

Additional restrictions apply to:

- Maximum height

- Eaves height near boundaries

- Overall garden coverage

Approval under the Larger Home Extension Scheme is not automatic and may require formal prior approval from the local authority.

## 3. Double-Storey Rear Extensions (Restricted Category)

A double-storey rear extension is more limited under PD and is only potentially achievable if:

- It extends no more than 3 metres beyond the original rear wall

- It is at least 7 metres from the rear boundary

- It does not exceed the height of the existing dwelling

- Side-facing upper windows are obscure glazed and non-opening below the prescribed height

In many urban plots, double-storey extensions frequently require full planning permission.

## 4. Side Extensions (Highly Conditional)

A side extension may be possible under PD only where:

- It is single-storey

- It does not exceed half the width of the original dwelling

- It does not project beyond the principal elevation

- Height limits are complied with (particularly near boundaries)

If the extension projects forward of the principal elevation or materially alters the street-facing appearance, planning permission is typically required.

Side extensions are commonly subject to stricter scrutiny and may require formal planning approval depending on design and visibility.

# Important Clarification

Permitted Development is rule-based legislation.

If a proposal complies fully with all statutory limits, neighbour objections alone do not remove PD rights.

However:

- Non-compliance with any single condition may require full planning permission.

- Conservation areas, Article 4 Directions, listed building status or historic planning conditions may remove or restrict PD rights.

- Detailed design drawings are required to determine compliance.

- Building Regulations approval is always required separately.

# Professional Recommendation

Before commencing works, it is strongly recommended that:

- A detailed compliance assessment is carried out by a qualified professional, and

- A Lawful Development Certificate (LDC) is obtained from the local authority to formally confirm PD compliance.

## Official Guidance

For full legislative detail, please refer to Permitted development rights for householders - Technical Guidance: https://assets.publishing.service.gov.uk/media/5d77afc8e5274a27cdb2c9e9/190910_Tech_Guide_for_publishing.pdf